A.	SMOKE ORIGIN:

| Unit Number(s) | District Forest | Legal Descr | Owner Class | Elev | Acres | Tons | Ign time | Date Burned |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N/A | NEO | 4N 42 & 43E | Private | 3500 | 300 |  | 0900 | 11/3/10 |

B.	IMPACT DESCRIPTION:

| 1. | Area Affected | Enterprise/Wallowa Valley | SSRA | (Yes No ) |
| --- | --- | --- | --- | --- |

| 2. | Date | 11/04 | Time | 12:00pm | smoke entered area. | Duration | 18 hours |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 3. | Type: | Main Plume | Drift Smoke | Residual Smoke |
| --- | --- | --- | --- | --- |

4.	Describe Smoke Behavior (including distances and elevations of base of plume):  	The base of the smoke plume was at approximately 3500 feet and 25 miles to the NW of Enterprise

5.	Cause of intrusion/incident:  Burning of logging slash piles on private forest ownership

6.	Public complaints received:  2 from the Enterprise area

C.	FORECAST AND INSTRUCTIONS:

1.	Forecast transport wind direction and speed at ignition time and for next 12 hours:  SE to SSE 10-22 bcmg SE-S 8-14

2.	Observed transport wind direction and speed at ignition time and for next 12 hours: North/5 mph

3.	Forecast surface wind direction and speed at ignition time and for next 12 hours (24 hours if residual smoke was a factor):  SE to S 9-15 bcmg ESE to S 6-10

4.	Observed surface wind direction and speed at ignition time and for next 12 (24) hours:  North/0-5mph

5.	Describe significant changes in transport or surface wind conditions:  Possible inversion over night

Were these changes forecast  Yes

6.	Describe general weather conditions observed during the burn period and for the next 6 hours (sky conditions, type and height of clouds, precipitation etc).  Clear no precipitation

7.	If observed weather was different than the forecast, was Salem consulted?  No

8.	What were Smoke Management Instructions?  Include written and/or verbal  Unfavorable burning situation due to very poor smoke dispersion.  Delay ignitions until 10:00am.  Avoid ignitions withing 20 miles to the ESE through SSW of SSRA's.  For units that will smolder significantly through the night avoid burning within at least 35 miles to the ESE through SSW in or near dranages leading to SSRA's.  Recommend against burning units that will smolder significantly overnight.  Complete ignitions by 3:00pm.

D.	FUEL MOISTURES AT IGNITION TIME:

| 1 hour Unk | 10 hours Unk | 100 hours Unk | 1,000 hours Unk |
| --- | --- | --- | --- |

E.	OTHER VISIBILITY RESTRICTING SOURCES PRESENT:

| Field Smoke | Resident Emissions | Ag Smoke | Dust |
| --- | --- | --- | --- |

| Other prescribed Fire Smoke (source) |  | Other (Specify) |  |
| --- | --- | --- | --- |

| Wildfire Smoke (Fire’s Name) |  | Unable to identify |  |
| --- | --- | --- | --- |

F. 	EXPLAIN SPECIFICALLY THE CAUSE OF THE IMPACT.  The majority of the smoke intrusion was a result of logging slash pile being burned approximately 25 miles to the NW of Enterprise.

G.	COMMENTS:

SECTION H THROUGH M TO BE COMPLETED BY SALEM FORECASTER:

H.	IMPACT INTENSITY:

1.	Average SSRA prevailing visibility for 3 hours prior to start of impact 10 miles.

2.	Lowest prevailing visibility during duration of impact 3 miles.

3.	Average SSRA nephelometer for 3 hours prior to start of impact est .70

4.	Highest nephelometer during duration of impact 8.14

5.	Classification based on visibility or nephelometer:

Light 	Moderate 	Heavy 	Measured  or Estimated

Unknown or can't determine   	No classification due to other sources

If moderate or heavy, the number of hours in those categories:  Moderate 15   Heavy 3

I.	OBSERVED MIXING DEPTH, TRANSPORT WIND AND WINDSHEAR AT NEAREST UPPER AIR SITE.  No nearby upper air site.

J.	GENERAL SYNOPTIC CONDITIONS, BOTH LARGE AND SMALL SCALE.  Be as specific as possible with feature location.  Include surface and upper air map type.  Upper air charts show a broad ridge with its axis stretching from Arizona northward to Alberta and a broad trough in the Gulf of Alaska and the eastern Pacific about 900 miles west of the coast.  Upper air flow over Oregon was from the southwest.  Very stable conditions with warm air aloft giving very poor dispersion.

K.	WERE FORECASTS ADEQUATE?  (Yes  / No )  Why The forecast accurately captured the general synoptic conditions for the region but were wrong on low level wind direction.  None the less they highlighted the poor conditions with Marginal Smoke Dispersal Conditions headlined.

L.	WERE INSTRUCTIONS ADEQUATE?  (Yes  / No )  Why The instructions lead with the sentence, "Unfavorable burning situation due to very poor smoke dispersion." which proved to be accurate.  Forecast wind directions were not accurate, however,  leading to instructions that recommended burning be curtailed to the ESE through SSW.  Apparently light northerly winds pushed smoke into the Enterprise area.

- COMMENTS

Microscale and mesoscale wind direction forecasting in areas of complex terrain is quite difficult.  This is a situation where access to the University of Washington modeling information may have helped improve the forecast.  We will never know.

| Steve Meyer |  | Jim Little |
| --- | --- | --- |
| District/Forest Representative |  | Smoke Management Forecaster |
|  |  | 10-06 |
|  |  | Intrusion/Incident No. |