Section A:

Lower Sheep RX , Umatilla NF, Walla Walla RD  , 2000 acres. Dates burned 9/29-10/01

Approx 600 ac. Burned 09/29, approx 1350 acres burned 09/30, approx 50 acres  burned 10/01

4N 40E Sec 24,25,35,36; 3N 40E sec. 1,2,11,12

Unit ranged from 2200-4000’ elevation.  Estimated tonnage= 24k

Section B

1-3)Area affected Grande Ronde Valley (GR)  which is a SSRA..drift smoke entered valley on late evening of 09/29 remained in area through evening of 10/02

4)  For 09/29-30 Smoke generally rose 1000-2000 feet during day, moved North with surface influences, then south-southwest with transport influences. This movement of smoke was very slow as transport winds were light. On 10/01 a definite southerly direction existed for transport winds, this day small portion of unit were ignited to fortify containment lines (approx. 50-80 acres), although unburned portions of unit from previous two days of ignition were also actively burning.  Each night (29-01) ceiling lowered to less than 1000’  and smoke filled GR canyon towards GR valley.

5) Less than optimal transport winds  and dispersion conditions.

6) There were phone calls that were made to local FS offices, with concerns relating to the smoke in the valley.

Section C.

1) N-NE, 6-10, mph.

2) S-SW for first 3 hrs of ignition on 09/29, then N-NE for remainder of that day.  On 09/30, similar conditions existed. On 10/01, transport winds were S-SW 6-10 mph.

3) Upslope, S-SE 3-5 mph

4) Upslope,  SE-SW 2-5 mph

5) As ignition began transport winds were southerly in direction, as day progressed,  transport shifted to a northerly direction.  Yes, these conditions were forecasted.

6) Sky conditions were clear,  surface winds upslope, RH’s in the 20-30’s throughout day.

7) Yes, Salem was consulted on Friday, 10/01.  This discussion with Forecaster was regarding the need to do finishing touches on ignition for containment purposes (for approx. 50-80 acres) . Favorable S-SW transport winds existed on unit, these remained in place throughout day.

8) Avoid ignitions within 15 miles of SSRA, and 30 miles for units that will smolder (09/29) this changed to 20 and 35 miles by 10/01.

Section D.

1 hour= 8%, 10 hour= 10-12%, 100 hour= 12-14%, 1000 hour= 15-20%

Section E.

Other visibility sources seen via helicopter on 09/29 were some field burns in GR Valley. (approx. 2-3)

Section F.

Less than ideal transport winds